#### First of all we need to make 100% sure that the system is up to date. SSH into your rpi and type the following commands:

sudo apt update

sudo apt full-upgrade

#### Klipper Installation

Follow the instructions to install Klipper if you havent already.

- klipper: https://www.klipper3d.org/Installation.html

Installation Requirements

Start by making sure that the following configuration options are written in your printer.cfg file:

[virtual_sdcard]

path:

~/.octoprint/uploads/

[pause_resume]

[display_status]

SSH into your rpi with something like putty. In the terminal window type/copy and paste the following:

sudo nano /etc/default/klipper

Change the script so it looks like the following. This points klipper to the printer_config directory that you are about to make.

# Configuration for /etc/init.d/klipper

KLIPPY_USER=pi KLIPPY_EXEC=/home/pi/klippy-env/bin/python KLIPPY_ARGS="/home/pi/klipper/klippy/klippy.py /home/pi/printer_config/printer.cfg -l /tmp/klippy.log -a /tmp/klippy_uds"

Once you have done this press Ctrl + x to prompt exit file and then press y to save the changes.

Now create a folder called printer_config in /home/pi and move your printer.cfg file to the directory, then move into the newly created printer_config directory:

cd /home/pi

mkdir printer_config

mv printer.cfg printer_config

cd printer_config

Now create a file called moonraker.conf in this directory:

sudo nano /home/pi/printer_config/moonraker.conf

Copy this text to the file for your basic moonraker config:

# Sample Moonraker Configuration File

[server]

# Bind server defaults of 0.0.0.0, port 7125

enable_debug_logging: True

[file_manager]

config_path: /home/pi/printer_config

[database]

database_path: ~/.moonraker_database

[authorization]

enabled: True

trusted_clients:

127.0.0.1

force_logins: False

# Enter your client IP here or range here

cors_domains:

# Allow CORS requests for Fluidd

http://app.fluidd.xyz

# Enable Octoprint compatibility for Slicer uploads Supports Cura,

# Slic3r, and Slic3r dervivatives (PrusaSlicer, SuperSlicer)

[octoprint_compat]

[update_manager KlipperScreen]

type: git_repo

path: ~/KlipperScreen

origin: https://github.com/jordanruthe/KlipperScreen.git

env: ~/.KlipperScreen-env/bin/python

requirements: scripts/KlipperScreen-requirements.txt

install_script: scripts/KlipperScreen-install.sh

If you need extra config for moonraker then you can read the documentation here:

https://moonraker.readthedocs.io/en/latest/configuration/

Now we can clone the moonraker git repository with these commands:

cd

git clone https://github.com/Arksine/moonraker.git

Then run the install with the following command- (I could only seem to get moonraker working properly by installing it in the printer_config directory. In the /home/pi directory I would get a error message about permissions) :

cd ~/moonraker/scripts ./install-moonraker.sh -f -c /home/pi/printer_config/moonraker.conf

Now moonraker should be working.

Reboot the system for good measures:

sudo reboot now

Now we can carry on and install klipperscreen with the following command:

cd ~/git clone https://github.com/jordanruthe/KlipperScreen.gitcd ~/KlipperScreen./scripts/KlipperScreen-install.sh

Now we are going to modify some files. Type the following command:

sudo nano /boot/config.txt

Scroll to the bottom of the file and add these lines (only add display_rotate if you are wanting to rotate your screen display. Options are from 0-4 from memory. You will need to reboot after the change to make sure it has rotated to where you want it)

Press ctrl+x and then y to save.

Then copy/paste this command:

sudo nano boot/cmdline.txt

At the end of the line after “rootwait” copy/paste the following:

splash quiet plymouth.ignore-serial-consoles logo.nologo vt.global_cursor_default=0

It should look something like this:

Now ensure that xinput and xserver and x11 is installed:

sudo apt-get install libx11-dev libxext-dev libxi-dev x11proto-input-dev

wget http://github.com/downloads/tias/xinput … 7.5.tar.gz

sudo apt install xinput

Then

sudo apt install xserver-xorg-input-evdev

If you need to rotate the screen then run the following command to find the name of your screen:

DISPLAY=0 xinput –-list

You should see something like this:

(Notice that my screen has a typo in the name “ STMicroelectronics KEDEI_touchscreem” instead of “touchscreen”........ That held me up for quite some time lol.)

Now type the following but replace my screens name with the name of your device:

DISPLAY=0 xinput –list-props “STMicroelectronics KEDEI touchscreem”

You should see something like this:

Take note of the factory coordinate transformation matrix. Now type this command to rotate the matrix (you may need a different matrix combination than mine depending on your screen. Just have a search around for combinations and you will find one eventually):

DISPLAY=0 xinput set-prop “STMicroelectronics KEDEI_touchscreem” ‘Coordinate Transformation Matrix’ 0 –1 1 1 0 0 0 0 1

Test the touch matrix to check that it is correct for the screen oreintation.

Once satisfied then run the following command and add these lines:

sudo nano /etc/udev/rules.d/51-touchscreen.rules

Add the following line, changing your screen name and matrix to suit.

ACTION=="add", ATTRS{name}== “STMicroelectronics KEDEI_touchscreem”, ENV{LIBINPUT_CALIBRATION_MATRIX}="0 –1 1 1 0 0 0 0 1 "

Press ctrl+x and then y to save.

Also modify this file and add this line:

sudo nano /etc/rc.local

NOTE:

#### Touchscreen Calibration

Most people don't need to calibrate, but if you do need to calibrate your touchscreen, follow the below steps.

sudo add-apt-repository ppa:tias/xinput-calibrator-ppa

sudo apt-get install xinput_calibrator

Run this command:

DISPLAY=:0 xinput_calibrator --list

It will output something such as:

Device "wch.cn USB2IIC_CTP_CONTROL" id=6

Find the ID of your display and put it in the following command:

DISPLAY=:0 xinput_calibrator -v --device <id from last command>

Now install omxplayer:

Sudo apt install omxplayer

Now you can make your boot screen splash video. I used davinci resolve because to make mine because its free. There are plenty of tutorials on youtube.

Next we need to transfer our video to the rpi. Im unsure how to do this via terminal so I used winscp to move the video file to /home/pi (named hevort.mov):

Now we can confirm the splash screen is working by running this command in the terminal:

omxplayer hevort.mov

All going well the splash screen should play. Now we can modify one more file to make the splash screen play on boot:

sudo nano /etc/rc.local

Add this line at the bottom of the page before exit0:

omxplayer /home/pi/hevort.mov &

Ctrl+x and y to save.

Congratulations! You should now have a working (maybe rotated) touchscreen with your own custom splash video!!!